GROUP NUMBER: 11

ROLL NUMBER 1: 200260008		NAME 1: Anurag Abhijit Pendse
ROLL NUMBER 2: 20B030011		NAME 2: Aswin Suresh

TITLE:

Modelling a 3-body active system using Arduino

ABSTRACT:
In this project, we model a 3-body actively driven system using Arduino. The Arduino produces the noise inputs which we need to supply the system as well as carry out the required computations as needed to solve the coupled system of differential equations. The output is seen on a DSO as well as in a real time plot of all 3 particles on Microsoft Excel.

PROJECT DETAILS:     
The project deals with solving the system of coupled differential equations which model a 3- body system of actively driven interacting particles with Brownian Noise (η)

ⅆx1ⅆt=η+αx1+γx2+x3

ⅆx2ⅆt=η+αx2+γx1+x3

ⅆx3ⅆt=η+αx3+γx1+x2

ⅆy1ⅆt=η+αy1+γy2+y3

ⅆy2ⅆt=η+αy2+γy1+y3

ⅆy3ⅆt=η+αy3+γy1+y2

The values used for the parameters are as follows:

γ=0.3

α is a combination of a simple harmonic attraction as well as a random self-drive to counter it.

We set

<α>=-0.1

While a common  η has been written in all 6 equations, the values of η aren’t necessarily same for all 6 at a given time. This has been taken care of in the code where we are sampling the noise at different times for different coordinates.

To view all the particles simultaneously, we are using the PLX-DAQ library to write the coordinates to excel in real time to plot them.

The RC circuit used is as follows:

The outputs for various duty cycles are as follows:

Arduino 1: (Done by Anurag)

Arduino 2: (Done by Aswin)

Other than this, Anurag built the required circuits for converting the PWM output to an analog voltage. Aswin wrote the code to print values to Excel in real time. Both of us worked on obtaining suitable values for the parameters which gave a stable system.

MAIN COMPONENTS NEEDED TO BUILD THE PROJECT:

2 Arduinos, a breadboard, 12 resistors, 12 capacitors and a DSO are required to build the project.

RESULTS:

Using the design as specified above, we can observe the path the particle follows. The path appears to tally with what a random walk should look like. Furthermore, we can see that the path followed by a particular particle when looked at as confined within the box bounded [-2.5,2.5]X[-2.5,2.5] with periodic boundary conditions appears to be space filling. This tallies with expectations of the nature of the path (DOI: 10.1103/PhysRevE.87.042136).

https://drive.google.com/file/d/1VXDTxzpSW4ZKI-8aVeH_sBpfOTNgYA0h/view?usp=share_link

Output on Excel: https://drive.google.com/file/d/1xLORWKbZz_7Js30jniKJcTbHGEJfyRLa/view?usp=sharing

Output on DSO:

https://drive.google.com/file/d/1st0ec8QbnQ_1sgwmdLaT2NbmYkuRxsXa/view?usp=sharing

https://drive.google.com/file/d/1g8CH7KGgDnRfq8yMoJBHCS_-kOtnIkng/view?usp=sharing

APPENDIX:

Arduino 1:

int x1;

int x2;

int x3;

int x4;

int x5;

int x;

int y=0;

int alph1;

int alph;

int eta;

void setup() {

Serial.begin(9600);

pinMode(5,OUTPUT);

pinMode(6,OUTPUT);

}

void loop() {

alph1=unigen();

alph=int(alph1*y/1023);

eta=gaussiangen();

analogWrite(5,eta);

analogWrite(6,alph1);

Serial.println(eta);

delay(100);

}

int gaussiangen(){      \\Gaussian Noise(eta)

x1=random(0,256);

x2=random(0,256);

x3=random(0,256);

x4=random(0,256);

x5=random(0,256);

x=int((x1+x2+x3+x4+x5)/5);

return x ;

}

int unigen(){   \\Noise to be used added to alpha

int x1=random(0,256)

return x1;

}

Arduino 2:

#include<stdio.h>

#include<math.h>

int InputPin_1 = A0;

int InputPin_2 = A1;

const double gamma = 0.3;

const double step_time = 1;

double x1 = 3, x2 = 2, x3 = 3, y1 = 3, y2 = 2.5, y3 = 2;

void setup() {

Serial.begin(9600);

pinMode(InputPin_1, INPUT);

pinMode(InputPin_2, INPUT);

pinMode(5,OUTPUT);

pinMode(6,OUTPUT);

Serial.println("CLEARDATA");

Serial.println("LABEL,x1,y1,x2,y2,x3,y3");

Serial.println("RESSETTIMER");

}

double f(double p1, double p2, double p3, double N, double u){

return N + u*p1 + gamma*(p2 + p3);

}

//  double RK1(double f(double x, double y), double x0, double y0, double x, double h){ \\Runge-Kutta Integration

//      double y,k1,k2;

//      while(fabs(x-x0)>0.0000000001){

//          k1=h*f(x0,y0);

//          k2=h*f(x0+h/2.0,y0+k1/2.0);

//          y=y0+k2;

//          y0=y;

//          x0=x0+h;

//      }

//      return y;

//  }

//

//  double RK2(double f(double x, double y), double x0, double y0, double x, double h){

//      double y,k1,k2,k3,k4;

//      while(fabs(x-x0)>0.0000000001){

//          k1=h*f(x0,y0);

//          k2=h*f(x0+h/2.0,y0+k1/2.0);

//          k3=h*f(x0+h/2.0,y0+k2/2.0);

//          k4=h*f(x0+h,y0+k3);

//          y=y0+1/6.0*(k1+2*k2+2*k3+k4);

//          y0=y;

//          x0=x0+h;

//      }

//      return y;

double euler(double p1, double p2, double p3, double N, double u){  \\Euler integration

if (p1 > 4.99)

{p1=-4.5 + step_time*f(p1, p2, p3, N, u);}

else if (p1 < -4.99)

{p1=4.5 +step_time*f(p1, p2, p3, N, u);}

else

{p1=p1+step_time*f(p1, p2, p3, N, u);}

return p1;

}

double fullupdate(double p1, double p2, double p3){

int N1=analogRead(InputPin_1);

int u1=analogRead(InputPin_2);

double N = -1+(N1*2/1024);

double u =-0.1+(u1/1024);

p1=euler(p1,p2,p3,N,u);

return p1;

}

void loop() {

Serial.print("DATA,");

\\Serial.println("x1");

\\Serial.println(255*(x1+5)/10);

\\Serial.println("y1");

\\Serial.println(255*(y1+5)/10);

x1=fullupdate(x1,x2,x3);

delay(20);

x2=fullupdate(x2,x3,x1);

delay(20);

x3=fullupdate(x3,x1,x2);

delay(20);

y1=fullupdate(y1,y2,y3);

delay(20);

y2=fullupdate(y2,y3,y1);

delay(20);

y3=fullupdate(y3,y1,y2);

analogWrite(5,255*(x1+5)/10); \\ give PWM output

analogWrite(6,255*(y1+5)/10);

Serial.print(x1); \\Write to excel

Serial.print(",");

Serial.print(y1);

Serial.print(",");

Serial.print(x2);

Serial.print(",");

Serial.print(y2);

Serial.print(",");

Serial.print(x3);

Serial.print(",");

Serial.print(y3);

Serial.println("");

}